# GridSensai Literature TLDR (Source Summaries)

## NERC Long-Term Reliability Assessment

Evaluates whether North America’s power grid will remain reliable as electricity demand grows.

Key findings:

• Data center and AI compute demand is becoming one of the fastest-growing electricity drivers.

• Transmission congestion and capacity limits are emerging planning challenges.

• Utilities must expand transmission, generation, and planning tools to maintain reliability margins.

• Load forecasting is becoming more difficult because hyperscale facilities can appear suddenly and require extremely large power capacity.

Implication: Large loads are becoming strategic planning risks rather than simple local distribution issues.

## IEA Electricity Reports

Studies global electricity demand trends.

Key findings:

• Data centers and AI clusters are expected to significantly increase electricity demand.

• AI training facilities can require hundreds of megawatts per site.

• In some regions data center electricity demand could double within a decade.

Implication: Large computing infrastructure is becoming a structural driver of grid expansion.

## EPRI Data Center Energy and Grid Integration Research

Research on how hyperscale data centers interact with power systems.

Key findings:

• Data centers usually maintain stable baseline demand.

• Internal systems such as UPS and backup generation affect how facilities behave during disturbances.

• Grid planners often lack detailed models of internal facility electrical architecture.

• Communication between data center engineers and grid planners is often inconsistent.

Implication: Data centers are complex electrical systems that planners frequently model only in simplified form.

## Brattle Group Interconnection Queue Studies

Analysis of interconnection queues across U.S. power markets.

Key findings:

• Interconnection queues are highly congested.

• Engineering studies and transmission upgrade analysis often take years.

• Many projects withdraw after study phases due to upgrade costs or delays.

• Incomplete project data frequently slows engineering analysis.

Implication: The interconnection process is slowed mainly by engineering study complexity and incomplete modeling data.

## FERC Interconnection Reform Reports

Federal studies of delays in grid interconnection processes.

Key findings:

• Interconnection queues have grown dramatically.

• Project approval timelines frequently extend several years.

• Many projects fail or withdraw during the study process.

• Standardizing data submissions could significantly improve study timelines.

Implication: Regulatory reforms aim to accelerate interconnection through improved processes and standardized data requirements.

## PJM Manual 14G (Interconnection Planning Manual)

Explains how PJM evaluates large load and generation interconnections.

Applicants typically must provide:

• Electrical one‑line diagrams

• Load forecasts and operating characteristics

• Protection system descriptions

• Equipment ratings

• Power system modeling data

Purpose: These inputs allow planners to run power flow, stability, and contingency simulations.

## NERC Transmission Planning Reliability Standards

Defines how transmission planners must ensure reliability.

Key requirements:

• System behavior must be simulated under contingency conditions.

• Models must represent dynamic behavior of loads and generation.

• Voltage stability and frequency response must be evaluated.

Implication: Accurate load models are essential for reliability studies.

## IEEE Research on Data Center Power Management

Academic research examining how computing infrastructure consumes electricity.

Key findings:

• High‑performance computing clusters can change power demand rapidly.

• AI workloads may create synchronized demand changes across servers.

• Power management software can scale power consumption within seconds.

Implication: Large computing facilities may behave very differently from traditional industrial loads.

## EPRI Grid Planning Modernization Studies

Research focused on improving grid planning tools and processes.

Key recommendations:

• Standardized modeling templates for large loads.

• Digital tools for exchanging electrical system data.

• Improved collaboration between developers and planners.

• Automation of engineering study preparation.

Implication: Traditional planning processes were designed for industrial loads and may not handle hyperscale facilities efficiently.

## ISO Planning Documentation

Operational documentation used by grid operators.

Key findings:

• Interconnection studies rely on standardized modeling inputs.

• Inconsistent or incomplete project data slows engineering analysis.

• Standard templates for modeling data could reduce study timelines.