# Section3 Information Requested

Question

What electrical or operational data do transmission planners typically request from large load applicants?

Extracted Answer

Transmission planners typically request peak demand forecasts, hourly or seasonal load profiles, electrical one‑line diagrams, protection schemes, and equipment specifications. These inputs allow planners to run power flow and stability simulations to determine how the new load will affect the transmission network.

Source

FERC Order No. 2023; PJM Manual 14G; EPRI Data Center Power Demand Research

Evidence or Key Passage

Planning manuals specify that applicants must provide electrical diagrams and load forecasts so planners can run power flow simulations.

Source Type

ISO/utility documentation

Confidence Level

High

Notes

Most ISOs request similar core modeling data.

Question

What types of modeling information are needed to evaluate a large load connection?

Extracted Answer

Planners typically require steady‑state power flow models, short‑circuit models, and dynamic models that describe how the load behaves during disturbances such as voltage dips or frequency deviations.

Source

NERC Transmission Planning Reliability Standards; IEEE Power System Planning Literature

Evidence or Key Passage

Reliability standards require modeling of system behavior under contingency conditions.

Source Type

Industry standard

Confidence Level

High

Notes

Dynamic modeling becomes more important for very large loads.