GridSensai Research Extraction Template

Section Title

Information Data Centers Can Provide vs Cannot Provide (Section 4)

Question

21. What information are you generally comfortable providing to utilities during interconnection studies?

Extracted Answer

Industry guidance indicates that data center developers are typically comfortable providing high‑level electrical and operational data needed for grid studies. This commonly includes expected peak demand, load growth forecasts, approximate commissioning schedules, electrical single‑line diagrams, protection concepts, and general descriptions of facility architecture. These inputs allow utilities to run power flow, stability, and short‑circuit studies without requiring disclosure of sensitive IT infrastructure details.

Source

NERC (2025) Reliability Guideline: Risk Mitigation for Emerging Large Loads

GridLab (2025) Large Loads Interim Report

ERCOT Large Load Interconnection Process Documentation

Evidence or Key Passage

• NERC guidance identifies common planning inputs including peak demand forecasts, ramp schedules, one‑line diagrams, and protection settings.

• GridLab describes applications typically including demand size, location, electrical design diagrams, and load ramp expectations.

Source Type

Industry reliability guideline / ISO documentation

Confidence Level

High

Notes

Utilities generally only require electrical system information relevant to grid modeling rather than internal IT design details.

Question

22. What types of information are difficult to provide because it does not exist yet during early planning?

Extracted Answer

Early in project development, many technical details of the facility are not finalized. Equipment vendors may not yet be selected, protection settings may be unknown, and detailed electrical designs may still be evolving. As a result, developers often provide preliminary estimates or placeholder modeling parameters during the early feasibility and system impact study phases.

Source

GridLab (2025) Large Loads Interim Report

Brattle Group & Berkeley Lab (2025) Large Loads: Interconnection, Tariff Designs, and State Actions

Evidence or Key Passage

• GridLab notes that early applications frequently rely on preliminary technical information because final equipment selections and designs are not complete.

• Brattle/Berkeley Lab highlight that planning processes must often operate with incomplete information during early development stages.

Source Type

Industry research report

Confidence Level

High

Notes

Dynamic models, protection settings, and detailed electrical equipment parameters are commonly unavailable early in development.

Question

23. What information is considered proprietary or sensitive and cannot easily be shared?

Extracted Answer

Data center developers often consider detailed IT architecture, workload management systems, server configurations, and internal operational algorithms to be proprietary. While these factors influence power consumption patterns, companies may avoid sharing detailed information because it could reveal competitive advantages or internal technology strategies.

Source

Industry analyses of hyperscale data center operations

NERC reports on emerging large loads

Evidence or Key Passage

• NERC documentation notes that operators may have confidentiality concerns regarding operational details when sharing information for planning studies.

Source Type

Industry analysis / reliability guidance

Confidence Level

Medium

Notes

Sensitivity concerns typically relate to computing architecture rather than electrical infrastructure.

Question

24. Are there concerns about revealing operational details that might expose competitive advantages or internal architecture?

Extracted Answer

Yes. Large technology companies often treat internal infrastructure design, workload scheduling algorithms, and computing architecture as confidential intellectual property. While utilities require electrical characteristics for planning studies, developers may limit disclosure of information that could reveal business strategy or proprietary engineering practices.

Source

NERC (2025) Emerging Large Load Reliability Guidance

Industry commentary on hyperscale infrastructure design

Evidence or Key Passage

• NERC guidance acknowledges confidentiality considerations when collecting operational information from large load operators.

Source Type

Reliability guideline / industry commentary

Confidence Level

Medium

Notes

Utilities typically focus on electrical characteristics rather than operational algorithms, which helps mitigate confidentiality concerns.

Question

25. Are there situations where your engineering team cannot provide the exact data utilities request?

Extracted Answer

Yes. Utilities sometimes request detailed modeling parameters or operational characteristics that may not be known until later phases of engineering design. In these cases, developers may provide estimated values, conservative assumptions, or ranges until more precise data becomes available during the detailed design phase.

Source

GridLab (2025) Large Loads Interim Report

Transmission planning guidance documents from utilities

Evidence or Key Passage

• GridLab indicates that some required modeling inputs are unavailable early in development and must be approximated during early study phases.

Source Type

Industry report

Confidence Level

High

Notes

This is particularly common for dynamic models, power electronics behavior, and protection system settings.

Question

26. If utilities request dynamic load models, how accurate can those models realistically be during the design phase?

Extracted Answer

Dynamic load models developed during early design phases are typically simplified approximations based on expected equipment types and operational assumptions. While these models can capture general system behavior, their accuracy improves as the project progresses and detailed equipment specifications become available.

Source

NERC reliability guidance on modeling emerging large loads

Academic research on power system load modeling

Evidence or Key Passage

• NERC notes that modeling large electronic loads requires assumptions about equipment behavior and control systems during early planning stages.

Source Type

Reliability guideline / academic research

Confidence Level

Medium

Notes

Model accuracy generally improves as vendor equipment data becomes available during detailed engineering.

Question

27. Do you ever feel utilities ask for information that is not practical to provide?

Extracted Answer

Some industry discussions suggest that utilities occasionally request data that is difficult to provide during early project stages, particularly highly detailed dynamic models or operational data that depends on final equipment configuration. However, most stakeholders recognize that these requests are driven by legitimate reliability concerns and the need for accurate grid studies.

Source

GridLab (2025) Large Loads Interim Report

Brattle Group & Berkeley Lab (2025) Large Loads: Interconnection, Tariff Designs, and State Actions

Evidence or Key Passage

• Industry reports note that utilities must balance planning accuracy with the reality that project details may not yet be finalized.

Source Type

Industry research report

Confidence Level

Medium

Notes

Challenges often arise from timing differences between project development and grid planning requirements.